Micromouse Starter Robot Assembly Instructions

| Author | Binara Wasala |
| --- | --- |
| Last Revision | 12/07/2026 |
| Revision | A |

Parts:

| Part | Qty. |
| --- | --- |
| 3.7V LiPo Battery | 1 |
| 43mm Rubber Wheel | 2 |
| Caster Ball | 1 |
| IR Sensor | 3 |
| M3x8mm Hex Socket Bolt | 15 |
| Micro Motor 60RPM | 2 |
| MM Base Plate | 1 |
| MM Caster Ball Extension | 1 |
| MM Motor Clamp | 2 |
| MM PCB | 1 |
| MM Top Plate | 1 |

|  | Micromouse Starter Robot | Step: | 1 |
| --- | --- | --- | --- |
| Parts Required: - 1 x MM Base Plate - 1 x MM Caster Ball Extension - 2 x M3x8mm Hex Socket Bolt |  |  |  |
| Instructions: - Fasten the MM Caster Ball Extension (B) to the MM Base Plate (A) using M3x8mm bolts (C). |  |  |  |
| 2 x C B A |  |  |  |
|  | Micromouse Starter Robot | Step: | 2 |
| Parts Required: - 1 x MM Base Plate with components fitted (where step 1 of this procedure has been completed) - 1 x Caster Ball - 2 x M3x8mm Hex Socket Bolt |  |  |  |
| Instructions: - Fasten the Caster Ball (B) to the MM Base Plate (A) using M3x8mm bolts (C). |  |  |  |
| 2 x C A B |  |  |  |
|  | Micromouse Starter Robot | Step: | 3 |
| Parts Required: - 1 x MM Base Plate with components fitted (where steps 1 to 2 of this procedure have been completed) - 2 x MM Motor Clamp - 2 x Micro Motor 60RPM - 4 x M3x8mm Hex Socket Bolt |  |  |  |
| Instructions: - Place the Micro Motors (C) on the MM Base Plate (A) and secure using MM Motor Clamps (B). - Fasten the MM Motor Clamps (B) to the MM Base Plate (A) using M3x8mm bolts (D). - Route the cables from the Micro Motors (C) through the designated holes in the MM Base Plate (A). |  |  |  |
| 2 x B 4 x D 2 x C A |  |  |  |
|  | Micromouse Starter Robot | Step: | 4 |
| Parts Required: - 1 x MM Base Plate with components fitted (where steps 1 to 3 of this procedure have been completed) - 3 x IR Sensor - 3 x M3x8mm Hex Socket Bolt |  |  |  |
| Instructions: - Fasten the IR Sensors (B) to the MM Base Plate (A) using M3x8mm bolts (C). |  |  |  |
| 3 x B 3 x C A |  |  |  |
|  | Micromouse Starter Robot | Step: | 5 |
| Parts Required: - 1 x MM Base Plate with components fitted (where steps 1 to 4 of this procedure have been completed) - 1 x 3.7B LiPo Battery |  |  |  |
| Instructions: - Place the LiPo Battery (B) in the designated cutout in the MM Base Plate (A). |  |  |  |
| B A |  |  |  |
|  | Micromouse Starter Robot | Step: | 6 |
| Parts Required: - 1 x MM Base Plate with components fitted (where steps 1 to 5 of this procedure have been completed) - 1 x MM Top Plate - 1 x MM PCB - 4 x M3x8mm Hex Socket Bolt |  |  |  |
| Instructions: - Place the MM Top Plate (B) on the MM Base Plate (A). - Rotate the MM PCB (C) to the desired orientation, place on the MM Top Plate (B) and fasten using M3x8mm bolts (D). - Route the motor cables, battery cable, and IR sensor cables through the designated holes in the MM Top Plate (B) and connect accordingly to the MM PCB (C). |  |  |  |
| A 4 x D B C |  |  |  |
|  | Micromouse Starter Robot | Step: | 7 |
| Parts Required: - 1 x MM Base Plate with components fitted (where steps 1 to 6 of this procedure have been completed) - 2 x 43mm Rubber Wheel |  |  |  |
| Instructions: - Connect the 43mm Rubber Wheels (B) onto the motor shafts on the MM Base Plate (A) until flush. |  |  |  |
| A 2 x B |  |  |  |
|  | Micromouse Starter Robot | Step: | 8 |
| Parts Required: |  |  |  |
| Instructions: - Assembly Complete. |  |  |  |